International Training College

Wireless Network Solutions

Assignment 1

Kalahari Logistics Ltd – Wireless and WAN Network Design

FINAL REPORT

Date: 26 August 2026

| Student Name and Surname | Student Number |
| --- | --- |
| Raheem Kisting | ITCLIT240956 |
| Sven Plaatjes | ITCLIT220568 |
| Amon Theodor | ITCLIT220573 |
| Elenora Vahekeni | ITCLIT230097 |

Kalahari Logistics Ltd – Wireless and WAN Network Design

Executive Summary

This project involves designing and implementing a multi-site network for Kalahari Logistics Ltd, connecting the Windhoek headquarters (HQ) in Namibia with branch offices in Walvis Bay (Namibia), Johannesburg (South Africa), and a data centre in London (UK). Each site has its own LAN subnet, and Windhoek also provides a wireless (Wi-Fi) network for mobile clients. Cisco routers, switches, and an access point were configured with static IPv4 addressing and basic security. A dedicated SSID and WPA2-PSK encryption secure the wireless LAN. All configurations were verified by ping and traceroute tests across LANs and WAN links.

Group Members:

Member1 (Sven Plaatjes)

Member2 (Elenora Vahekeni)

Member3 (Amon Theodor)

Member4 (Raheem Kisting)

Objectives:

- Connect Windhoek HQ, Walvis Bay branch, Johannesburg branch, and London data centre into one unified network.

- Provide local LAN connectivity (routers → switches → PCs/servers) at each site.

- Enable wireless connectivity at Windhoek HQ with SSID KalahariLogistics (using WPA2-PSK security).

- Use static IPv4 addressing and configure routing so that hosts at any site can communicate with any other site.

- Document the design, configuration, testing, and provide screenshots in the final report.

Network Topology

The final topology includes a central router at Windhoek (WH-RTR) with serial links to branch routers at Walvis Bay (WB-RTR), Johannesburg (JHB-RTR) and London (LON-RTR). Each router’s GigabitEthernet0/0 interface connects to a local switch (WH-SW, WB-SW, JHB-SW, LON-SW) serving that site’s LAN. The Windhoek switch also connects to a server, PCs, and a wireless access point (WH-AP). Wireless laptops (WH-LAP01, WH-LAP02) join the Windhoek network via the AP. The logical layout is shown below:

WH_RTR[WH-RTR 10.0.0.1 / 192.168.10.1]

WH_SW[WH-SW 192.168.10.2]

WH_AP[WH-AP 192.168.10.3]

WH_PC1[WH-PC01 192.168.10.10]

WH_PC2[WH-PC02 192.168.10.11]

WH_SERVER[WH-Server 192.168.10.12]

WH_LAP1[Laptop1 192.168.10.20]

WH_LAP2[Laptop2 192.168.10.21]

WH_SW --- WH_PC1

WH_SW --- WH_PC2

WH_SW --- WH_SERVER

WH_SW --- WH_AP

WH_SW --- WH_RTR

WH_AP --- WH_LAP1

WH_AP --- WH_LAP2

WH_RTR -- 10.0.0.2 -- WB_RTR[WB-RTR 10.0.0.2 / 192.168.30.1]

WB_RTR -- WB_SW[WB-SW 192.168.30.2]

WB_SW -- WB_PC1[WB-PC01 192.168.30.10]

WB_SW -- WB_PC2[WB-PC02 192.168.30.11]

WH_RTR -- 10.0.0.6 -- JHB_RTR[JHB-RTR 10.0.0.6 / 192.168.40.1]

JHB_RTR -- JHB_SW[JHB-SW 192.168.40.2]

JHB_SW -- JHB_PC1[JHB-PC01 192.168.40.10]

JHB_SW -- JHB_PC2[JHB-PC02 192.168.40.11]

WH_RTR -- 10.0.0.10 -- LON_RTR[LON-RTR 10.0.0.10 / 192.168.50.1]

LON_RTR -- LON_SW[LON-SW 192.168.50.2]

LON_SW -- LON_PC1[LON-PC01 192.168.50.10]

LON_SW -- LON_Server[LON-Server 192.168.50.11]

Figure 1: Example multi-site network topology. Each branch router (WB-RTR, JHB-RTR, LON-RTR) connects to WH-RTR at Windhoek. Switches (SW) connect local hosts, and a wireless AP (WH-AP) provides Wi-Fi. Cisco routers route traffic between subnets, and switches/APs serve the local LANs.

IP Addressing Plan

All four sites use distinct private IPv4 subnets. The static address allocations are:

- Windhoek HQ: Subnet 192.168.10.0/24. Default gateway = 192.168.10.1 (WH-RTR). Hosts: PCs (.10, .11), Server (.12), AP (.3), Laptops (.20, .21), Switch MGMT (.2).

- Walvis Bay: Subnet 192.168.30.0/24. Gateway = 192.168.30.1 (WB-RTR). Hosts: PCs (.10, .11), Switch MGMT (.2).

- Johannesburg: Subnet 192.168.40.0/24. Gateway = 192.168.40.1 (JHB-RTR). Hosts: PCs (.10, .11), Switch MGMT (.2).

- London: Subnet 192.168.50.0/24. Gateway = 192.168.50.1 (LON-RTR). Hosts: PC (.10), Server (.11), Switch MGMT (.2).

Each branch link to Windhoek uses a /30 subnet to minimize wasted addresses:

- WH–WB: 10.0.0.0/30 (WH-RTR=10.0.0.1, WB-RTR=10.0.0.2).

- WH–JHB: 10.0.0.4/30 (WH-RTR=10.0.0.5, JHB-RTR=10.0.0.6).

- WH–LON: 10.0.0.8/30 (WH-RTR=10.0.0.9, LON-RTR=10.0.0.10).

This plan ensures each router interface and host has a unique IP in the correct subnet.

Device Configuration Summaries

Routers (WH-RTR, WB-RTR, JHB-RTR, LON-RTR): Each router was named appropriately (e.g. WH-RTR) and set with secure passwords (enable secret WNSadmin123, console WNSconsole123, VTY WNSvty123). The GigabitEthernet0/0 interfaces were configured with the LAN IPs, and the Serial0/0/0 interfaces with the point-to-point WAN IPs, as per the table above. For example, on WH-RTR:

WH-RTR(config)# interface GigabitEthernet0/0

WH-RTR(config-if)# ip address 192.168.10.1 255.255.255.0

WH-RTR(config-if)# no shutdown

WH-RTR(config-if)# exit

WH-RTR(config)# interface Serial0/0/0

WH-RTR(config-if)# ip address 10.0.0.1 255.255.255.252

WH-RTR(config-if)# no shutdown

No clock rate was set on branch routers (DTE). All router configs were saved. We verified interface status and IPs with show ip interface brief; only the correct, activated interfaces showed up/up.

Switches (WH-SW, WB-SW, JHB-SW, LON-SW): Each switch was named (e.g. WH-SW) and secured. We assigned the management IP on VLAN 1 as the second IP of each subnet (e.g. 192.168.10.2/24 on WH-SW) and set the default-gateway to the router. Example on WB-SW:

WB-SW(config)# interface vlan 1

WB-SW(config-if)# ip address 192.168.30.2 255.255.255.0

WB-SW(config-if)# no shutdown

WB-SW(config-if)# exit

WB-SW(config)# ip default-gateway 192.168.30.1

This allows out-of-band management of the switch. All other switch ports remain in VLAN1 by default.

Wireless Access Point (WH-AP): The AP’s management IP was set to 192.168.10.3/24 (gateway 192.168.10.1). In the AP’s configuration (Port 1 settings), we defined:

- SSID: KalahariLogistics

- Security: WPA2-PSK (AES) with passphrase 12345678

These settings match recommended WPA2 Personal encryption. Figure 2 shows an example Packet Tracer wireless topology with clients attached to the AP.

Figure 2: Wireless connectivity in Packet Tracer. All laptops are shown connected to the AP (open wifi). In practice, our AP is secured with WPA2-PSK. Each laptop had a wireless NIC (WPC300N) installed and joined SSID KalahariLogistics successfully.

Verification and Testing

Local LAN Tests: We used ping from client PCs to their local router and peers. For example, from Windhoek PC1 (192.168.10.10):

- ping 192.168.10.1 (WH-RTR) – successful replies.

- ping 192.168.10.11 (Windhoek PC2) – successful replies.

- ping 192.168.10.12 (Windhoek Server) – successful.

All sites showed similar results, confirming LAN connectivity and correct IP settings.

Wireless Tests: After connecting WH-LAP01 and WH-LAP02 to the SSID, we gave them IPs 192.168.10.20 and .21. From WH-LAP01 we pinged the router:

WH-LAP01> ping 192.168.10.3   (AP)

WH-LAP01> ping 192.168.10.1   (Windhoek router)

Both succeeded, showing the wireless subnet is 192.168.10.0/24 and the clients can reach the gateway.

Cross-Site Tests: We then enabled routing (static routes on branches pointing to WH-RTR). From WH-PC01, we tested reachability of each remote LAN:

- ping 192.168.30.10 (Walvis Bay PC) – Success.

- ping 192.168.40.11 (Johannesburg PC) – Success.

- ping 192.168.50.11 (London Server) – Success.

Similarly, from a Walvis Bay PC we could ping Windhoek hosts (e.g. ping 192.168.10.12 worked). We also ran traceroute (tracert) to verify the path went through the intermediate routers as expected. Figure 4 shows an example cross-site test.

Figure 4: Example traceroute from Windhoek to another site (simulated). Each hop corresponds to an ISP interface or branch router, matching the configured serial links and subnets.

Testing Flowchart and Schedule: We followed a systematic approach:

- Configure all devices

- Test local LAN pings

- Test wireless connectivity

- Test cross-site pings/traceroutes

- Document results and fix issues

- Show code

Issues and Resolutions

No critical issues remained. We encountered a few common hiccups:

- Interfaces down/down: Initially many interfaces were down; issuing no shutdown on each fixed this.

- Misplaced VLAN: Ensured all important ports were in VLAN 1 so the management IP stayed reachable.

- Wireless pairing: Laptops did not connect until the wireless adapter (WPC300N) was enabled and the SSID/key were exactly correct.

- Typographical errors: Any mistyped IP/mask caused ping failure; careful review of configs resolved these quickly.

Conclusions and Recommendations

The completed network meets all project goals: every branch is connected, and Windhoek HQ has secured wireless. Key conclusions:

- Proper IP planning (distinct /24 networks per site and /30 WANs) avoids overlap.

- Verifying with show ip interface brief is essential – it clearly shows which interfaces are up.

- A clear topology diagram (Figure 1) aided understanding and communication of the design.

For improvements, one could implement a dynamic routing protocol (like OSPF) for scalability, rather than manual static routes. VLAN segmentation could also be introduced if more hosts or departments are added.

Appendix

Key Configuration Snippets

- Windhoek Router (WH-RTR): (Excerpt of running-config)

hostname WH-RTR

enable secret WNSadmin123

interface GigabitEthernet0/0

ip address 192.168.10.1 255.255.255.0

no shutdown

interface Serial0/0/0

ip address 10.0.0.1 255.255.255.252

no shutdown

- Windhoek Switch (WH-SW):

hostname WH-SW

enable secret WNSadmin123

interface vlan 1

ip address 192.168.10.2 255.255.255.0

no shutdown

ip default-gateway 192.168.10.1

- Windhoek AP (WH-AP): (Configuration summary)
SSID=KalahariLogistics, WPA2-PSK key=12345678, IP=192.168.10.3/24.

Deliverables Checklist

The following items are included in the GitHub repository:

- Packet Tracer File: Kalahari_Logistics_WNS.pkt – the complete simulation.

- Screenshots:

  - 01_complete_topology.png: Full Packet Tracer topology (an overview of all sites).

  - 02_WH-RTR_configuration.png … 05_LON-RTR_configuration.png: show ip interface brief on each router after config.

  - 06_WH-AP_configuration.png: WH-AP wireless settings (SSID, security, IP).

  - 07_WH_Wireless_Verification.png: Ping test from WH-LAP01 (wireless laptop) to router.

  - 08_CrossSite_Verification.png: Ping/traceroute results across sites (Windhoek↔branches).

- Report Files: WNS_Group_Assignment_Report.pdf (this document) and .docx source.

- Reflections: Individual member reflections (reflections/member1_reflection.md, etc.).

All outputs and evidence (configs, screenshots) are documented above and available in the repo.

REFERENCES

Breezy Codes. (2025, July 3). How to add a wireless access point into your network in CISCO Packet Tracer [Video]. YouTube. https://www.youtube.com/watch?v=thyLjcjrf4I

ButlerU Information Systems. (2020, April 9). WAN setup in Cisco Packet Tracer Review [Video]. YouTube. https://www.youtube.com/watch?v=YdM7yBrtAck

Eating With ZekkouKyo. (2024, April 1). Creating WAN in Cisco Packet Tracer [Video]. YouTube. https://www.youtube.com/watch?v=qtFdr_XDbcQ

GeeksforGeeks. (2026, April 28). Types of network PAN, LAN, WAN and MAN. GeeksforGeeks. https://www.geeksforgeeks.org/computer-networks/types-of-area-networks-lan-man-and-wan/

101Labs. (2026, June 24). Cisco Packet Tracer Full Tutorial for Beginners (2026) [Video]. YouTube. https://www.youtube.com/watch?v=9HB2mc7M8z0